DYNAMIC DISTILLATION  /  COMPLETED RUN

# C3C4_DD047_chemsep_duty_control_isolation_180s_r2

ChemSep_centered_condenser_duty_pressure_and_inventory_control_isolation

| RUN VALIDITY USABLE | DYNAMIC GATE PASS | FINAL SCORE 0.549 | SIM / WALL 0.494 |
| --- | --- | --- | --- |

# Run Identity

| Item | Value |
| --- | --- |
| Run ID | 20260711_163320 |
| Started | 2026-07-11 16:33:20 |
| Completed | 2026-07-11 16:39:24 |
| Run type | Continuation from native checkpoint |
| Input workbook | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_initializer_residual_vapor_state_stage2_20260706.xlsx |
| Input checkpoint | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd046_inventory_coordination_continue300s_20260711\c3c4_initializer_residual_vapor_state_stage2_20260706__checkpoint_20260711_160221.npz |
| Summary data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd047_chemsep_duty_control_isolation_180s_r2_20260711\column_summary_20260711_163320.csv |
| Tray-profile data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd047_chemsep_duty_control_isolation_180s_r2_20260711\column_profile_20260711_163320.csv |

# Operating Snapshot

| Parameter | Start | End | Units |
| --- | --- | --- | --- |
| Feed | 7,142.98 | 7,142.98 | lbmol/h |
| Distillate | 2,386.93 | 3,623.58 | lbmol/h |
| Bottoms | 4,761.98 | 8,176.58 | lbmol/h |
| Reflux | Not reported | 4,900.00 | lbmol/h |
| Top pressure | 222.619 | 224.680 | psia |
| Bottom pressure | 316.632 | 231.631 | psia |
| Distillate temperature | 113.43 | 113.43 | deg F |
| Bottoms temperature | 209.72 | 190.58 | deg F |
| Condenser duty | Not reported | -51.629 | MMBtu/h |
| Reboiler duty | Not reported | 54.844 | MMBtu/h |
| Distillate drum controller PV | Not reported | 52.10 | lbmol |
| Bottoms sump controller PV | Not reported | 47.66 | lbmol |
| Steady-state score | Not reported | 0.5491 | - |

## Product Composition

| Component | Distillate start | Distillate end | Bottoms start | Bottoms end |
| --- | --- | --- | --- | --- |
| n-Butane | 7.514% | 7.207% | 69.910% | 70.673% |
| n-Pentane | 0.164% | 0.129% | 15.571% | 15.609% |
| n-Propane | 92.322% | 92.664% | 14.519% | 13.719% |
| source | Not reported | Not reported | Not reported | Not reported |

# Dynamic Trends

# Final Tray Profiles

# Final Tray Profile Table

| Stage | T (deg F) | P (psia) | L (lbmol/h) | V (lbmol/h) | x n-Propane | x n-Butane | x n-Pentane | y n-Propane | y n-Butane | y n-Pentane |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 113.43 | 224.680 | 4,900.0 | 0.0 | 0.9365 | 0.0633 | 0.0003 | 0.0000 | 0.0000 | 0.0000 |
| 2 | 123.02 | 226.680 | 4,908.8 | 8,341.8 | 0.7967 | 0.2015 | 0.0018 | 0.9377 | 0.0621 | 0.0002 |
| 3 | 133.94 | 226.885 | 4,939.4 | 8,047.4 | 0.6574 | 0.3402 | 0.0024 | 0.8587 | 0.1408 | 0.0005 |
| 4 | 145.12 | 227.100 | 4,999.5 | 7,783.4 | 0.5323 | 0.4643 | 0.0034 | 0.7706 | 0.2285 | 0.0008 |
| 5 | 154.66 | 227.326 | 5,089.0 | 7,613.8 | 0.4389 | 0.5563 | 0.0048 | 0.6885 | 0.3102 | 0.0013 |
| 6 | 161.69 | 227.563 | 5,199.5 | 7,532.5 | 0.3785 | 0.6147 | 0.0068 | 0.6239 | 0.3740 | 0.0021 |
| 7 | 166.49 | 227.810 | 5,319.5 | 7,503.0 | 0.3434 | 0.6469 | 0.0097 | 0.5787 | 0.4181 | 0.0032 |
| 8 | 169.76 | 228.065 | 5,436.8 | 7,495.0 | 0.3250 | 0.6609 | 0.0141 | 0.5484 | 0.4466 | 0.0050 |
| 9 | 172.16 | 228.327 | 5,541.9 | 7,493.2 | 0.3160 | 0.6629 | 0.0210 | 0.5281 | 0.4640 | 0.0079 |
| 10 | 174.24 | 228.594 | 5,629.9 | 7,486.5 | 0.3103 | 0.6572 | 0.0325 | 0.5137 | 0.4735 | 0.0128 |
| 11 | 176.57 | 228.865 | 5,699.9 | 7,461.2 | 0.3040 | 0.6439 | 0.0521 | 0.5016 | 0.4772 | 0.0213 |
| 12 | 179.82 | 229.207 | 12,889.1 | 7,405.7 | 0.3033 | 0.6105 | 0.0862 | 0.4896 | 0.4740 | 0.0364 |
| 13 | 182.97 | 229.554 | 12,926.6 | 7,573.8 | 0.2818 | 0.6304 | 0.0879 | 0.4570 | 0.5043 | 0.0387 |
| 14 | 186.77 | 229.909 | 12,957.0 | 7,616.6 | 0.2559 | 0.6537 | 0.0904 | 0.4170 | 0.5412 | 0.0418 |
| 15 | 191.16 | 230.272 | 12,982.1 | 7,659.8 | 0.2275 | 0.6784 | 0.0941 | 0.3702 | 0.5836 | 0.0462 |
| 16 | 195.99 | 230.591 | 13,005.2 | 7,720.8 | 0.1985 | 0.7015 | 0.0999 | 0.3195 | 0.6280 | 0.0525 |
| 17 | 201.09 | 230.918 | 13,024.5 | 7,795.7 | 0.1716 | 0.7191 | 0.1092 | 0.2682 | 0.6696 | 0.0622 |
| 18 | 206.36 | 231.270 | 13,042.6 | 7,877.4 | 0.1497 | 0.7269 | 0.1233 | 0.2209 | 0.7018 | 0.0773 |
| 19 | 212.04 | 231.631 | 13,058.9 | 7,951.2 | 0.1652 | 0.7200 | 0.1149 | 0.1818 | 0.7182 | 0.1000 |
| 20 | 219.21 | 231.631 | 5,028.8 | 8,030.0 | 0.1060 | 0.7345 | 0.1594 | 0.2022 | 0.7109 | 0.0870 |

# Simulation Configuration

| Parameter | Value |
| --- | --- |
| Runtime Mode | hydraulic |
| Thermo Mode | dwsim |
| Dwsim Property Package | pr |
| Integrator | explicit-euler |
| N Steps | 900 |
| Dt Sec | 0.2 |
| Log Every N Steps | 25 |
| Thermo Every N Steps | 1 |
| Include Energy | True |
| Enable Equilibrium Relaxation | True |
| Equilibrium Relaxation Mode | composition-exponential |
| Equilibrium Tau Sec | 0.5 |
| Flash Feed At Stage Conditions | True |
| Enable Liquid Hydraulic Override | True |
| Liquid Hydraulic Model | francis |
| Liquid Hydraulic Override Alpha | 0.25 |
| Enable Level Control | True |
| Top Level Pv Mode | true-level |
| Top Level Kc | 20.0 |
| Top Level Ti Sec | 120.0 |
| Bottom Level Pv Mode | true-level |
| Bottom Level Kc | 3.0 |
| Bottom Level Ti Sec | 300.0 |
| Enable Pressure Control | True |
| Pressure Control Mv | condenser-duty |
| Top Pressure Sp Psia | 222.6194497041178 |
| Top Pressure Kc | -150000.0 |
| Top Pressure Ti Sec | 300.0 |
| Top Pressure Pv Filter Tau Sec | 5.0 |
| Top Pressure Mv Slew Limit Per S | 25000.0 |
| Condenser Duty Mode | specified |
| Condenser Duty Btu Per H | -49500000.0 |
| Condenser Duty Min Btu Per H | -52000000.0 |
| Condenser Duty Max Btu Per H | -46000000.0 |
| Vapor Holdup Relaxation Sec | 0.0 |
| Vapor Flow Relaxation Sec | 0.0 |
| Vapor Flow Zero Temperature Target | True |
| Dynamic Vflow Nominal Hi Ratio | 1.05 |
| Steady State Window Sec | 30.0 |
| Steady State Min Time Sec | 60.0 |
| Steady State Rel State Rate Tol Per S | 0.003 |
| Steady State Temp Rate Tol F Per S | 0.15 |
| Steady State Sp Error Tol | 0.02 |
| Steady State Require Sp | False |

## Exact Launch Command

"C:\Users\Thomas Zvolensky\anaconda3\python.exe" -m dynamic_distillation.dynamic_run_scaffold_v1 --excel logs\c3c4_initializer_residual_vapor_state_stage2_20260706.xlsx --init-from-checkpoint .\logs\c3c4_dd046_inventory_coordination_continue300s_20260711\c3c4_initializer_residual_vapor_state_stage2_20260706__checkpoint_20260711_160221.npz --run-name C3C4_DD047_chemsep_duty_control_isolation_180s_r2 --run-description ChemSep_centered_condenser_duty_pressure_and_inventory_control_isolation --runtime-mode hydraulic --thermo dwsim --dwsim-property-package pr --include-energy --equilibrium-relaxation-mode composition-exponential --equilibrium-tau-sec 0.5 --equilibrium-component-transfer-max-cancel-multiplier 1.0 --flash-feed-at-stage-conditions --enable-liquid-hydraulic-override --liquid-hydraulic-model francis --liquid-hydraulic-override-alpha 0.25 --enable-level-control --ignore-workbook-level-pv-mode --top-level-pv-mode true-level --bottom-level-pv-mode true-level --top-level-kc 20 --top-level-ti 120 --bottom-level-kc 3 --bottom-level-ti 300 --enable-pressure-control --pressure-control-mv condenser-duty --top-pressure-sp 222.6194497041178 --top-pressure-kc -150000 --top-pressure-ti 300 --top-pressure-pv-filter-tau-sec 5 --top-pressure-mv-slew-limit-per-s 25000 --condenser-duty-mode specified --condenser-duty-btuph -49500000 --condenser-duty-min-btuph -52000000 --condenser-duty-max-btuph -46000000 --enable-top-drum-resident-condensation --top-drum-resident-condensation-tau-sec 30 --top-drum-resident-condensation-max-frac-per-step 0.10 --reflux 4900 --vapor-holdup-relaxation-sec 0 --vapor-flow-relaxation-sec 0 --vapor-flow-zero-temperature-target --use-excel-vapor-holdup --dynamic-vflow-nominal-hi-ratio 1.05 --disable-startup-thermo-conditioning --disable-restart-reentry-settling --n-steps 900 --dt 0.2 --log-every 25 --logs-dir logs\c3c4_dd047_chemsep_duty_control_isolation_180s_r2_20260711 --allow-repeat-command --no-word-report