DYNAMIC DISTILLATION  /  COMPLETED RUN

# C3C4_DD049_chemsep_reflux_product_split_180s_r3

ChemSep_reflux_and_product_split_with_top_level_integral_rebase

| RUN VALIDITY REVIEW | DYNAMIC GATE NOT PASSED | FINAL SCORE 2.302 | SIM / WALL 0.283 |
| --- | --- | --- | --- |

AUTOMATED ASSESSMENT  The dynamic acceptance gate did not pass (final score 2.302). Top pressure is 2.61 psi from its logged target, so the rate gate does not establish acceptable pressure control.

# Run Identity

| Item | Value |
| --- | --- |
| Run ID | 20260711_214317 |
| Started | 2026-07-11 21:43:17 |
| Completed | 2026-07-11 21:53:53 |
| Run type | Continuation from native checkpoint |
| Input workbook | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_initializer_residual_vapor_state_stage2_20260706.xlsx |
| Input checkpoint | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd048_selective_integral_rebase_180s_r2_20260711\c3c4_initializer_residual_vapor_state_stage2_20260706__checkpoint_20260711_170234.npz |
| Summary data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd049_chemsep_reflux_product_split_180s_r3_20260711\column_summary_20260711_214317.csv |
| Tray-profile data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd049_chemsep_reflux_product_split_180s_r3_20260711\column_profile_20260711_214317.csv |

# Operating Snapshot

| Parameter | Start | End | Units |
| --- | --- | --- | --- |
| Feed | 7,142.98 | 7,142.98 | lbmol/h |
| Distillate | 2,386.93 | 2,364.10 | lbmol/h |
| Bottoms | 4,761.98 | 4,815.21 | lbmol/h |
| Reflux | Not reported | 5,967.32 | lbmol/h |
| Top pressure | 222.619 | 225.234 | psia |
| Bottom pressure | 316.632 | 232.421 | psia |
| Distillate temperature | 113.43 | 113.43 | deg F |
| Bottoms temperature | 199.75 | 204.85 | deg F |
| Condenser duty | Not reported | -49.967 | MMBtu/h |
| Reboiler duty | Not reported | 54.844 | MMBtu/h |
| Distillate drum controller PV | Not reported | 51.98 | lbmol |
| Bottoms sump controller PV | Not reported | 50.65 | lbmol |
| Steady-state score | Not reported | 2.3017 | - |

## Product Composition

| Component | Distillate start | Distillate end | Bottoms start | Bottoms end |
| --- | --- | --- | --- | --- |
| n-Butane | 6.958% | 6.770% | 71.514% | 72.656% |
| n-Pentane | 0.101% | 0.080% | 15.810% | 16.331% |
| n-Propane | 92.941% | 93.150% | 12.676% | 11.014% |
| source | Not reported | Not reported | Not reported | Not reported |

# Dynamic Trends

# Final Tray Profiles

# Final Tray Profile Table

| Stage | T (deg F) | P (psia) | L (lbmol/h) | V (lbmol/h) | x n-Propane | x n-Butane | x n-Pentane | y n-Propane | y n-Butane | y n-Pentane |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 113.43 | 225.234 | 5,967.3 | 0.0 | 0.9375 | 0.0623 | 0.0002 | 0.0000 | 0.0000 | 0.0000 |
| 2 | 123.02 | 227.234 | 5,932.1 | 8,537.4 | 0.8221 | 0.1768 | 0.0011 | 0.9375 | 0.0623 | 0.0002 |
| 3 | 133.94 | 227.502 | 5,823.1 | 8,283.1 | 0.7023 | 0.2963 | 0.0014 | 0.8582 | 0.1415 | 0.0003 |
| 4 | 145.12 | 227.769 | 5,645.8 | 8,057.2 | 0.5877 | 0.4104 | 0.0019 | 0.7699 | 0.2296 | 0.0005 |
| 5 | 154.66 | 228.034 | 5,438.5 | 7,912.3 | 0.4915 | 0.5059 | 0.0026 | 0.6875 | 0.3117 | 0.0009 |
| 6 | 161.69 | 228.297 | 5,249.3 | 7,839.3 | 0.4167 | 0.5794 | 0.0038 | 0.6226 | 0.3761 | 0.0013 |
| 7 | 166.49 | 228.557 | 5,110.6 | 7,776.9 | 0.3601 | 0.6340 | 0.0059 | 0.5768 | 0.4211 | 0.0021 |
| 8 | 169.76 | 228.816 | 5,031.2 | 7,729.7 | 0.3169 | 0.6735 | 0.0096 | 0.5457 | 0.4509 | 0.0034 |
| 9 | 172.16 | 229.075 | 5,002.7 | 7,683.0 | 0.2829 | 0.7007 | 0.0164 | 0.5245 | 0.4698 | 0.0057 |
| 10 | 174.24 | 229.337 | 5,011.0 | 7,631.2 | 0.2559 | 0.7151 | 0.0290 | 0.5091 | 0.4809 | 0.0100 |
| 11 | 176.57 | 229.601 | 5,043.7 | 7,566.5 | 0.2362 | 0.7111 | 0.0528 | 0.4961 | 0.4857 | 0.0182 |
| 12 | 179.82 | 229.945 | 12,221.8 | 7,481.2 | 0.2714 | 0.6406 | 0.0880 | 0.4857 | 0.4802 | 0.0340 |
| 13 | 182.97 | 230.296 | 12,265.5 | 7,618.6 | 0.2455 | 0.6652 | 0.0894 | 0.4518 | 0.5126 | 0.0356 |
| 14 | 186.77 | 230.654 | 12,316.3 | 7,638.5 | 0.2152 | 0.6935 | 0.0913 | 0.4102 | 0.5522 | 0.0376 |
| 15 | 191.16 | 231.023 | 12,372.4 | 7,670.6 | 0.1824 | 0.7238 | 0.0939 | 0.3605 | 0.5991 | 0.0404 |
| 16 | 195.99 | 231.348 | 12,439.4 | 7,724.3 | 0.1483 | 0.7538 | 0.0979 | 0.3057 | 0.6502 | 0.0440 |
| 17 | 201.09 | 231.683 | 12,508.0 | 7,796.3 | 0.1153 | 0.7803 | 0.1043 | 0.2476 | 0.7031 | 0.0493 |
| 18 | 206.36 | 232.045 | 12,583.0 | 7,882.8 | 0.0851 | 0.7989 | 0.1160 | 0.1895 | 0.7526 | 0.0578 |
| 19 | 212.04 | 232.421 | 12,660.7 | 7,965.0 | 0.0556 | 0.8045 | 0.1398 | 0.1328 | 0.7941 | 0.0731 |
| 20 | 219.21 | 232.421 | 4,630.6 | 8,030.0 | 0.0331 | 0.7785 | 0.1884 | 0.0686 | 0.8196 | 0.1118 |

# Simulation Configuration

| Parameter | Value |
| --- | --- |
| Runtime Mode | hydraulic |
| Thermo Mode | dwsim |
| Dwsim Property Package | pr |
| Integrator | explicit-euler |
| N Steps | 900 |
| Dt Sec | 0.2 |
| Log Every N Steps | 25 |
| Thermo Every N Steps | 1 |
| Include Energy | True |
| Enable Equilibrium Relaxation | True |
| Equilibrium Relaxation Mode | composition-exponential |
| Equilibrium Tau Sec | 0.5 |
| Flash Feed At Stage Conditions | True |
| Enable Liquid Hydraulic Override | True |
| Liquid Hydraulic Model | francis |
| Liquid Hydraulic Override Alpha | 0.25 |
| Enable Level Control | True |
| Top Level Pv Mode | true-level |
| Top Level Kc | 20.0 |
| Top Level Ti Sec | 120.0 |
| Bottom Level Pv Mode | true-level |
| Bottom Level Kc | 3.0 |
| Bottom Level Ti Sec | 300.0 |
| Enable Pressure Control | True |
| Pressure Control Mv | condenser-duty |
| Top Pressure Sp Psia | 222.6194497041178 |
| Top Pressure Kc | -150000.0 |
| Top Pressure Ti Sec | 300.0 |
| Top Pressure Pv Filter Tau Sec | 5.0 |
| Top Pressure Mv Slew Limit Per S | 25000.0 |
| Condenser Duty Mode | specified |
| Condenser Duty Btu Per H | -49500000.0 |
| Condenser Duty Min Btu Per H | -52000000.0 |
| Condenser Duty Max Btu Per H | -46000000.0 |
| Vapor Holdup Relaxation Sec | 0.0 |
| Vapor Flow Relaxation Sec | 0.0 |
| Vapor Flow Zero Temperature Target | True |
| Dynamic Vflow Nominal Hi Ratio | 1.05 |
| Steady State Window Sec | 30.0 |
| Steady State Min Time Sec | 60.0 |
| Steady State Rel State Rate Tol Per S | 0.003 |
| Steady State Temp Rate Tol F Per S | 0.15 |
| Steady State Sp Error Tol | 0.02 |
| Steady State Require Sp | False |

## Exact Launch Command

"C:\Users\Thomas Zvolensky\anaconda3\python.exe" -m dynamic_distillation.dynamic_run_scaffold_v1 --excel logs\c3c4_initializer_residual_vapor_state_stage2_20260706.xlsx --init-from-checkpoint .\logs\c3c4_dd048_selective_integral_rebase_180s_r2_20260711\c3c4_initializer_residual_vapor_state_stage2_20260706__checkpoint_20260711_170234.npz --rebase-controller-integral top-level --run-name C3C4_DD049_chemsep_reflux_product_split_180s_r3 --run-description ChemSep_reflux_and_product_split_with_top_level_integral_rebase --runtime-mode hydraulic --thermo dwsim --dwsim-property-package pr --include-energy --equilibrium-relaxation-mode composition-exponential --equilibrium-tau-sec 0.5 --flash-feed-at-stage-conditions --enable-liquid-hydraulic-override --liquid-hydraulic-model francis --liquid-hydraulic-override-alpha 0.25 --enable-level-control --ignore-workbook-level-pv-mode --top-level-pv-mode true-level --bottom-level-pv-mode true-level --top-level-kc 20 --top-level-ti 120 --bottom-level-kc 3 --bottom-level-ti 300 --enable-pressure-control --pressure-control-mv condenser-duty --top-pressure-sp 222.6194497041178 --top-pressure-kc -150000 --top-pressure-ti 300 --top-pressure-pv-filter-tau-sec 5 --top-pressure-mv-slew-limit-per-s 25000 --condenser-duty-mode specified --condenser-duty-btuph -49500000 --condenser-duty-min-btuph -52000000 --condenser-duty-max-btuph -46000000 --enable-top-drum-resident-condensation --reflux 5967.322711692588 --vapor-holdup-relaxation-sec 0 --vapor-flow-relaxation-sec 0 --vapor-flow-zero-temperature-target --use-excel-vapor-holdup --dynamic-vflow-nominal-hi-ratio 1.05 --disable-startup-thermo-conditioning --disable-restart-reentry-settling --n-steps 900 --dt 0.2 --log-every 25 --logs-dir logs\c3c4_dd049_chemsep_reflux_product_split_180s_r3_20260711 --allow-repeat-command