DYNAMIC DISTILLATION  /  COMPLETED RUN

# C3C4_DD054_composition_settle_continue300s

Unchanged_continuation_for_lower_column_composition_front_settling

| RUN VALIDITY USABLE | DYNAMIC GATE PASS | FINAL SCORE 0.420 | SIM / WALL 0.560 |
| --- | --- | --- | --- |

# Run Identity

| Item | Value |
| --- | --- |
| Run ID | 20260712_101442 |
| Started | 2026-07-12 10:14:42 |
| Completed | 2026-07-12 10:23:38 |
| Run type | Continuation from native checkpoint |
| Input workbook | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_initializer_residual_vapor_state_stage2_20260706.xlsx |
| Input checkpoint | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd053_unchanged_operating_point_hold_300s_20260712\c3c4_initializer_residual_vapor_state_stage2_20260706__checkpoint_20260712_092336.npz |
| Summary data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd054_composition_settle_continue300s_20260712\column_summary_20260712_101442.csv |
| Tray-profile data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_dd054_composition_settle_continue300s_20260712\column_profile_20260712_101442.csv |

# Operating Snapshot

| Parameter | Start | End | Units |
| --- | --- | --- | --- |
| Feed | 7,142.98 | 7,142.98 | lbmol/h |
| Distillate | 2,386.93 | 1,999.48 | lbmol/h |
| Bottoms | 4,761.98 | 4,902.47 | lbmol/h |
| Reflux | Not reported | 5,967.32 | lbmol/h |
| Top pressure | 222.619 | 221.098 | psia |
| Bottom pressure | 316.632 | 232.235 | psia |
| Distillate temperature | 113.43 | 113.43 | deg F |
| Bottoms temperature | 204.67 | 211.02 | deg F |
| Condenser duty | Not reported | -50.338 | MMBtu/h |
| Reboiler duty | Not reported | 54.844 | MMBtu/h |
| Distillate drum controller PV | Not reported | 51.77 | lbmol |
| Bottoms sump controller PV | Not reported | 49.46 | lbmol |
| Steady-state score | Not reported | 0.4201 | - |

## Product Composition

| Component | Distillate start | Distillate end | Bottoms start | Bottoms end |
| --- | --- | --- | --- | --- |
| n-Butane | 6.479% | 6.426% | 77.498% | 77.823% |
| n-Pentane | 0.035% | 0.025% | 17.148% | 16.092% |
| n-Propane | 93.485% | 93.549% | 5.354% | 6.085% |
| source | Not reported | Not reported | Not reported | Not reported |

# Dynamic Trends

# Final Tray Profiles

# Final Tray Profile Table

| Stage | T (deg F) | P (psia) | L (lbmol/h) | V (lbmol/h) | x n-Propane | x n-Butane | x n-Pentane | y n-Propane | y n-Butane | y n-Pentane |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 113.43 | 221.098 | 5,967.3 | 0.0 | 0.9365 | 0.0634 | 0.0001 | 0.0000 | 0.0000 | 0.0000 |
| 2 | 123.13 | 223.098 | 5,967.3 | 8,173.6 | 0.8296 | 0.1700 | 0.0004 | 0.9365 | 0.0634 | 0.0001 |
| 3 | 133.94 | 223.534 | 5,967.3 | 7,946.5 | 0.7153 | 0.2840 | 0.0007 | 0.8570 | 0.1429 | 0.0002 |
| 4 | 145.12 | 223.980 | 5,967.3 | 7,741.7 | 0.6110 | 0.3878 | 0.0013 | 0.7688 | 0.2308 | 0.0004 |
| 5 | 154.66 | 224.439 | 5,967.2 | 7,592.4 | 0.5299 | 0.4680 | 0.0022 | 0.6867 | 0.3124 | 0.0008 |
| 6 | 161.69 | 224.908 | 5,967.1 | 7,520.4 | 0.4734 | 0.5230 | 0.0037 | 0.6223 | 0.3761 | 0.0016 |
| 7 | 166.49 | 225.385 | 5,966.9 | 7,495.2 | 0.4356 | 0.5584 | 0.0060 | 0.5775 | 0.4197 | 0.0028 |
| 8 | 169.76 | 225.867 | 5,966.7 | 7,485.1 | 0.4103 | 0.5797 | 0.0099 | 0.5477 | 0.4475 | 0.0048 |
| 9 | 172.16 | 226.353 | 5,966.5 | 7,475.3 | 0.3912 | 0.5924 | 0.0164 | 0.5286 | 0.4634 | 0.0080 |
| 10 | 174.24 | 226.842 | 5,966.5 | 7,457.6 | 0.3750 | 0.5978 | 0.0272 | 0.5142 | 0.4724 | 0.0134 |
| 11 | 176.57 | 227.334 | 5,966.4 | 7,422.7 | 0.3579 | 0.5968 | 0.0453 | 0.5025 | 0.4752 | 0.0222 |
| 12 | 179.82 | 227.944 | 13,109.5 | 7,359.9 | 0.3249 | 0.5928 | 0.0823 | 0.4903 | 0.4726 | 0.0371 |
| 13 | 182.97 | 228.563 | 13,109.5 | 7,501.7 | 0.3007 | 0.6155 | 0.0838 | 0.4571 | 0.5037 | 0.0392 |
| 14 | 186.77 | 229.193 | 13,109.5 | 7,537.9 | 0.2721 | 0.6422 | 0.0858 | 0.4164 | 0.5417 | 0.0419 |
| 15 | 191.16 | 229.837 | 13,109.3 | 7,592.0 | 0.2402 | 0.6714 | 0.0884 | 0.3683 | 0.5861 | 0.0455 |
| 16 | 195.99 | 230.407 | 13,108.5 | 7,664.2 | 0.2062 | 0.7015 | 0.0923 | 0.3154 | 0.6341 | 0.0505 |
| 17 | 201.09 | 230.990 | 13,106.9 | 7,750.7 | 0.1727 | 0.7284 | 0.0988 | 0.2604 | 0.6818 | 0.0578 |
| 18 | 206.36 | 231.609 | 13,103.6 | 7,846.2 | 0.1438 | 0.7467 | 0.1095 | 0.2081 | 0.7221 | 0.0699 |
| 19 | 212.04 | 232.235 | 13,097.8 | 7,932.5 | 0.1511 | 0.7471 | 0.1018 | 0.1652 | 0.7454 | 0.0894 |
| 20 | 219.21 | 232.235 | 5,067.8 | 8,030.0 | 0.0971 | 0.7619 | 0.1411 | 0.1852 | 0.7378 | 0.0770 |

# Simulation Configuration

| Parameter | Value |
| --- | --- |
| Runtime Mode | hydraulic |
| Thermo Mode | dwsim |
| Dwsim Property Package | pr |
| Integrator | explicit-euler |
| N Steps | 1500 |
| Dt Sec | 0.2 |
| Log Every N Steps | 50 |
| Thermo Every N Steps | 1 |
| Include Energy | True |
| Enable Equilibrium Relaxation | True |
| Equilibrium Relaxation Mode | composition-exponential |
| Equilibrium Tau Sec | 0.5 |
| Flash Feed At Stage Conditions | True |
| Enable Liquid Hydraulic Override | True |
| Liquid Hydraulic Model | francis |
| Liquid Hydraulic Override Alpha | 0.25 |
| Enable Level Control | True |
| Top Level Pv Mode | true-level |
| Top Level Kc | 20.0 |
| Top Level Ti Sec | 120.0 |
| Bottom Level Pv Mode | true-level |
| Bottom Level Kc | 3.0 |
| Bottom Level Ti Sec | 300.0 |
| Enable Pressure Control | True |
| Pressure Control Mv | condenser-duty |
| Top Pressure Sp Psia | 222.6194497041178 |
| Top Pressure Kc | -300000.0 |
| Top Pressure Ti Sec | 180.0 |
| Top Pressure Pv Filter Tau Sec | 5.0 |
| Top Pressure Mv Slew Limit Per S | 25000.0 |
| Condenser Duty Mode | specified |
| Condenser Duty Btu Per H | -51600000.0 |
| Condenser Duty Min Btu Per H | -55000000.0 |
| Condenser Duty Max Btu Per H | -46000000.0 |
| Vapor Holdup Relaxation Sec | 0.0 |
| Vapor Flow Relaxation Sec | 0.0 |
| Vapor Flow Zero Temperature Target | True |
| Dynamic Vflow Nominal Hi Ratio | 1.05 |
| Steady State Window Sec | 30.0 |
| Steady State Min Time Sec | 60.0 |
| Steady State Rel State Rate Tol Per S | 0.003 |
| Steady State Temp Rate Tol F Per S | 0.15 |
| Steady State Sp Error Tol | 0.02 |
| Steady State Require Sp | False |

## Exact Launch Command

"C:\Users\Thomas Zvolensky\anaconda3\python.exe" -m dynamic_distillation.dynamic_run_scaffold_v1 --excel logs\c3c4_initializer_residual_vapor_state_stage2_20260706.xlsx --init-from-checkpoint .\logs\c3c4_dd053_unchanged_operating_point_hold_300s_20260712\c3c4_initializer_residual_vapor_state_stage2_20260706__checkpoint_20260712_092336.npz --run-name C3C4_DD054_composition_settle_continue300s --run-description Unchanged_continuation_for_lower_column_composition_front_settling --runtime-mode hydraulic --thermo dwsim --dwsim-property-package pr --include-energy --equilibrium-relaxation-mode composition-exponential --equilibrium-tau-sec 0.5 --flash-feed-at-stage-conditions --enable-liquid-hydraulic-override --liquid-hydraulic-model francis --liquid-hydraulic-override-alpha 0.25 --enable-level-control --ignore-workbook-level-pv-mode --top-level-pv-mode true-level --bottom-level-pv-mode true-level --top-level-kc 20 --top-level-ti 120 --bottom-level-kc 3 --bottom-level-ti 300 --enable-pressure-control --pressure-control-mv condenser-duty --top-pressure-sp 222.6194497041178 --top-pressure-kc -300000 --top-pressure-ti 180 --top-pressure-pv-filter-tau-sec 5 --top-pressure-mv-slew-limit-per-s 25000 --condenser-duty-mode specified --condenser-duty-btuph -51600000 --condenser-duty-min-btuph -55000000 --condenser-duty-max-btuph -46000000 --enable-top-drum-resident-condensation --top-drum-resident-condensation-tau-sec 30 --top-drum-resident-condensation-max-frac-per-step 0.10 --reflux 5967.322711692588 --vapor-holdup-relaxation-sec 0 --vapor-flow-relaxation-sec 0 --vapor-flow-zero-temperature-target --use-excel-vapor-holdup --dynamic-vflow-nominal-hi-ratio 1.05 --disable-startup-thermo-conditioning --disable-restart-reentry-settling --n-steps 1500 --dt 0.2 --log-every 50 --logs-dir logs\c3c4_dd054_composition_settle_continue300s_20260712 --allow-repeat-command