DYNAMIC DISTILLATION  /  COMPLETED RUN

# core_v3_kc3m_compkc30k_feedT_plus5F_complete600s

Final 239.5 seconds completing 600-second dual-retuned plus-5F model-robustness trajectory with composition logged and no undeclared product gate

| RUN VALIDITY USABLE | DYNAMIC GATE PASS | FINAL SCORE 0.955 | SIM / WALL 21.552 |
| --- | --- | --- | --- |

# Run Identity

| Item | Value |
| --- | --- |
| Run ID | 20260830_204510 |
| Started | 2026-08-30 20:45:10 |
| Completed | 2026-08-30 20:55:11 |
| Run type | Fresh start |
| Input workbook | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\distillation_column_template_20stage_chemsep_warmer_feed_seed_20260323.xlsx |
| Input checkpoint | None |
| Summary data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\core_v3_kc3m_compkc30k_feedT_plus5F_complete600s_20260830\column_summary_20260830_204510.csv |
| Tray-profile data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\core_v3_kc3m_compkc30k_feedT_plus5F_complete600s_20260830\column_profile_20260830_204510.csv |

# Operating Snapshot

| Parameter | Start | End | Units |
| --- | --- | --- | --- |
| Feed | Not reported | Not reported | lbmol/h |
| Distillate | 2,557.36 | 2,617.94 | lbmol/h |
| Bottoms | 4,377.41 | 4,469.73 | lbmol/h |
| Reflux | Not reported | Not reported | lbmol/h |
| Top pressure | Not reported | Not reported | psia |
| Bottom pressure | Not reported | Not reported | psia |
| Distillate temperature | 120.92 | 121.01 | deg F |
| Bottoms temperature | 217.32 | 217.53 | deg F |
| Condenser duty | -52.667 | -52.501 | MMBtu/h |
| Reboiler duty | 54.706 | 54.706 | MMBtu/h |
| Distillate drum controller PV | 0.50 | 0.50 | lbmol |
| Bottoms sump controller PV | Not reported | Not reported | lbmol |
| Steady-state score | 2.9148 | 0.9554 | - |

## Product Composition

| Component | Distillate start | Distillate end | Bottoms start | Bottoms end |
| --- | --- | --- | --- | --- |
| n-Butane | 12.178% | 12.366% | 79.210% | 79.212% |
| n-Pentane | 0.004% | 0.005% | 17.315% | 17.437% |
| n-Propane | 87.818% | 87.630% | 3.475% | 3.351% |

# Dynamic Trends

# Final Tray Profiles

# Final Tray Profile Table

| Stage | T (deg F) | P (psia) | L (lbmol/h) | V (lbmol/h) | x n-Propane | x n-Butane | x n-Pentane | y n-Propane | y n-Butane | y n-Pentane |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Simulation Configuration

| Parameter | Value |
| --- | --- |
| N Steps | 479 |
| Dt Sec | 0.5 |