DYNAMIC DISTILLATION  /  COMPLETED RUN

# ui_restart_test

Simulation operating report

| RUN VALIDITY REVIEW | DYNAMIC GATE NOT PASSED | FINAL SCORE 164.422 | SIM / WALL 0.796 |
| --- | --- | --- | --- |

AUTOMATED ASSESSMENT  The dynamic acceptance gate did not pass (final score 164.422). The final tray snapshot reports 14 failed thermo flashes; interpret operating results as invalid until the thermo path is restored. Top pressure is 110.22 psi from its logged target, so the rate gate does not establish acceptable pressure control. At least one geometry-based vessel level remains more than five percentage points from setpoint. The bottom level loop reported molar holdup rather than a geometry-based level fraction.

# Run Identity

| Item | Value |
| --- | --- |
| Run ID | 20260820_203305 |
| Started | 2026-08-20 20:33:05 |
| Completed | 2026-08-20 20:33:43 |
| Run type | Continuation from native checkpoint |
| Input workbook | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\distillation_column_template_20stage_chemsep_warmer_feed_seed_20260323.xlsx |
| Input checkpoint | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_10s_restart_runtime_memory_20260706\c3c4_initializer_profile_coeff_trial1_20260601__checkpoint_20260706_134127.npz |
| Summary data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\ui_runs\20260820_203210_ui-restart-test\column_summary_20260820_203305.csv |
| Tray-profile data | C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\ui_runs\20260820_203210_ui-restart-test\column_profile_20260820_203305.csv |

# Operating Snapshot

| Parameter | Start | End | Units |
| --- | --- | --- | --- |
| Feed | 7,142.98 | 7,142.98 | lbmol/h |
| Distillate | 2,380.99 | 2,186.10 | lbmol/h |
| Bottoms | 4,761.98 | 4,761.98 | lbmol/h |
| Reflux | Not reported | 2,000.00 | lbmol/h |
| Top pressure | 220.440 | 110.220 | psia |
| Bottom pressure | 316.632 | 156.637 | psia |
| Distillate temperature | 113.43 | -74.78 | deg F |
| Bottoms temperature | 220.53 | 220.47 | deg F |
| Condenser duty | Not reported | -49.640 | MMBtu/h |
| Reboiler duty | Not reported | 54.706 | MMBtu/h |
| Distillate drum level | Not reported | 41.31 | % |
| Bottoms sump controller PV | Not reported | 794.13 | lbmol |
| Steady-state score | Not reported | 164.4219 | - |

## Product Composition

| Component | Distillate start | Distillate end | Bottoms start | Bottoms end |
| --- | --- | --- | --- | --- |
| n-Butane | 10.145% | 10.734% | 78.150% | 77.920% |
| n-Pentane | 0.001% | 0.005% | 15.916% | 15.689% |
| n-Propane | 89.854% | 89.261% | 5.934% | 6.391% |
| source | Not reported | Not reported | Not reported | Not reported |

# Dynamic Trends

# Final Tray Profiles

# Final Tray Profile Table

| Stage | T (deg F) | P (psia) | L (lbmol/h) | V (lbmol/h) | x n-Propane | x n-Butane | x n-Pentane | y n-Propane | y n-Butane | y n-Pentane |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -74.78 | 110.220 | 2,000.0 | 0.0 | 0.7371 | 0.2601 | 0.0029 | 0.0000 | 0.0000 | 0.0000 |
| 2 | 88.61 | 112.220 | 5,676.4 | 10,484.7 | 0.7795 | 0.2191 | 0.0014 | 0.7371 | 0.2601 | 0.0029 |
| 3 | 88.85 | 114.705 | 5,450.6 | 9,122.3 | 0.8038 | 0.1956 | 0.0005 | 0.6527 | 0.3399 | 0.0074 |
| 4 | 115.05 | 116.929 | 5,308.1 | 8,533.4 | 0.6852 | 0.3119 | 0.0028 | 0.6331 | 0.3566 | 0.0103 |
| 5 | 132.71 | 119.052 | 5,237.9 | 8,107.8 | 0.6398 | 0.3580 | 0.0023 | 0.5551 | 0.4301 | 0.0147 |
| 6 | 145.03 | 121.125 | 5,209.0 | 8,029.2 | 0.5950 | 0.4032 | 0.0018 | 0.5120 | 0.4689 | 0.0191 |
| 7 | 155.27 | 123.188 | 5,196.3 | 7,955.3 | 0.5322 | 0.4639 | 0.0039 | 0.5040 | 0.4745 | 0.0215 |
| 8 | 162.57 | 125.266 | 5,185.3 | 7,952.7 | 0.4960 | 0.4984 | 0.0056 | 0.4785 | 0.4955 | 0.0260 |
| 9 | 167.32 | 127.431 | 5,166.0 | 8,097.9 | 0.4719 | 0.5196 | 0.0085 | 0.4431 | 0.5234 | 0.0336 |
| 10 | 171.32 | 129.838 | 5,127.8 | 8,451.2 | 0.3778 | 0.5932 | 0.0289 | 0.3775 | 0.5739 | 0.0486 |
| 11 | 176.10 | 132.453 | 5,057.1 | 8,935.0 | 0.3361 | 0.6245 | 0.0394 | 0.3013 | 0.6326 | 0.0661 |
| 12 | 160.17 | 135.346 | 12,352.5 | 8,880.1 | 0.3256 | 0.5912 | 0.0831 | 0.2810 | 0.6467 | 0.0722 |
| 13 | 162.71 | 138.491 | 12,382.7 | 8,205.8 | 0.2954 | 0.6169 | 0.0877 | 0.2724 | 0.6525 | 0.0751 |
| 14 | 169.78 | 141.740 | 12,425.5 | 8,331.8 | 0.2820 | 0.6294 | 0.0886 | 0.2319 | 0.6835 | 0.0846 |
| 15 | 171.98 | 145.132 | 12,485.2 | 8,495.1 | 0.2466 | 0.6580 | 0.0954 | 0.2224 | 0.6901 | 0.0874 |
| 16 | 172.74 | 148.081 | 12,563.2 | 8,583.3 | 0.2292 | 0.6746 | 0.0962 | 0.1777 | 0.7216 | 0.1007 |
| 17 | 177.46 | 150.989 | 12,653.8 | 8,483.0 | 0.2009 | 0.6967 | 0.1024 | 0.1779 | 0.7211 | 0.1010 |
| 18 | 177.86 | 153.737 | 12,738.5 | 8,110.5 | 0.1748 | 0.7157 | 0.1095 | 0.1755 | 0.7219 | 0.1026 |
| 19 | 185.46 | 156.637 | 12,776.5 | 7,431.0 | 0.1602 | 0.7275 | 0.1122 | 0.1634 | 0.7273 | 0.1093 |
| 20 | 219.21 | 156.637 | 4,761.9 | 8,014.6 | 0.1602 | 0.7275 | 0.1122 | 0.1602 | 0.7275 | 0.1122 |

# Simulation Configuration

| Parameter | Value |
| --- | --- |
| Runtime Mode | hydraulic |
| Thermo Mode | clapeyron |
| Dwsim Property Package | pr |
| Integrator | explicit-euler |
| N Steps | 150 |
| Dt Sec | 0.2 |
| Log Every N Steps | 5 |
| Thermo Every N Steps | 1 |
| Include Energy | True |
| Enable Equilibrium Relaxation | True |
| Equilibrium Relaxation Mode | auto |
| Enable Level Control | False |
| Top Level Pv Mode | molar-holdup |
| Bottom Level Pv Mode | molar-holdup |
| Enable Pressure Control | False |
| Pressure Control Mv | auto |
| Condenser Duty Mode | total-condense |
| Vapor Flow Zero Temperature Target | False |
| Steady State Window Sec | 30.0 |
| Steady State Min Time Sec | 60.0 |
| Steady State Rel State Rate Tol Per S | 0.003 |
| Steady State Temp Rate Tol F Per S | 0.15 |
| Steady State Sp Error Tol | 0.02 |
| Steady State Require Sp | False |

## Exact Launch Command

"C:\Users\Thomas Zvolensky\anaconda3\python.exe" -m dynamic_distillation.dynamic_run_scaffold_v1 --excel "C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\distillation_column_template_20stage_chemsep_warmer_feed_seed_20260323.xlsx" --runtime-mode hydraulic --thermo clapeyron --include-energy --n-steps 150 --dt 0.2 --log-every 5 --run-name ui_restart_test --init-from-checkpoint "C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\c3c4_10s_restart_runtime_memory_20260706\c3c4_initializer_profile_coeff_trial1_20260601__checkpoint_20260706_134127.npz" --logs-dir "C:\Users\Thomas Zvolensky\Documents\Python Scripts\Dynamic_DistillationII\logs\ui_runs\20260820_203210_ui-restart-test" --allow-repeat-command